Camera feed code

#include "esp_camera.h"

#include <WiFi.h>

// ===========================

// Camera Pin Definitions for AI-Thinker ESP32-CAM

// ===========================

#define PWDN_GPIO_NUM     32

#define RESET_GPIO_NUM    -1

#define XCLK_GPIO_NUM      0

#define SIOD_GPIO_NUM     26

#define SIOC_GPIO_NUM     27

#define Y9_GPIO_NUM       35

#define Y8_GPIO_NUM       34

#define Y7_GPIO_NUM       39

#define Y6_GPIO_NUM       36

#define Y5_GPIO_NUM       21

#define Y4_GPIO_NUM       19

#define Y3_GPIO_NUM       18

#define Y2_GPIO_NUM        5

#define VSYNC_GPIO_NUM    25

#define HREF_GPIO_NUM     23

#define PCLK_GPIO_NUM     22

// ===========================

// Wi-Fi Access Point Credentials

// ===========================

const char* ssid = "ESP32-CAM-Live";

const char* password = "12345678";

// ===========================

// Web Server Setup on Port 80

// ===========================

WiFiServer server(80);

void setup() {

Serial.begin(115200);

Serial.setDebugOutput(true);

Serial.println();

// Configure and initialize the camera

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM;

config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM;

config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM;

config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM;

config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk = XCLK_GPIO_NUM;

config.pin_pclk = PCLK_GPIO_NUM;

config.pin_vsync = VSYNC_GPIO_NUM;

config.pin_href = HREF_GPIO_NUM;

config.pin_sscb_sda = SIOD_GPIO_NUM;

config.pin_sscb_scl = SIOC_GPIO_NUM;

config.pin_pwdn = PWDN_GPIO_NUM;

config.pin_reset = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_JPEG;

// Select frame size (higher = better quality but slower)

config.frame_size = FRAMESIZE_QVGA; // 320x240

config.jpeg_quality = 12;           // 0-63, lower is higher quality

config.fb_count = 1;                // Number of frame buffers to use

// Initialize the camera

esp_err_t err = esp_camera_init(&config);

if (err != ESP_OK) {

Serial.printf("Camera init failed with error 0x%x", err);

return;

}

// Start Access Point mode

WiFi.mode(WIFI_AP);

WiFi.softAP(ssid, password);

Serial.println("Access Point Started");

Serial.print("Connect to Wi-Fi: ");

Serial.println(ssid);

Serial.print("Then open: http://");

Serial.println(WiFi.softAPIP());

// Start the web server

server.begin();

}

void loop() {

// Check for a new client connecting to the server

WiFiClient client = server.available();

if (!client) {

return;

}

// Wait for data from the client to arrive

Serial.println("New Client Connected.");

while (client.connected() && !client.available()) {

delay(1);

}

// Read the first line of the HTTP request

String request = client.readStringUntil('\r');

Serial.println(request);

client.flush();

// The client is requesting the MJPEG stream

if (request.indexOf("/stream") != -1) {

// Send the correct HTTP headers for a video stream

client.println("HTTP/1.1 200 OK");

client.println("Content-Type: multipart/x-mixed-replace; boundary=frame");

client.println("Connection: close");

client.println();

// Continuously send JPEG frames

while (client.connected()) {

camera_fb_t * fb = esp_camera_fb_get();

if (!fb) {

Serial.println("Frame capture failed");

break;

}

client.printf("--frame\r\n");

client.printf("Content-Type: image/jpeg\r\n");

client.printf("Content-Length: %d\r\n\r\n", fb->len);

client.write(fb->buf, fb->len);

client.printf("\r\n");

esp_camera_fb_return(fb);

}

}

// If the request is for the root page ("/"), send the HTML interface

else {

// Send a simple HTML page

client.println("HTTP/1.1 200 OK");

client.println("Content-Type: text/html");

client.println("Connection: close");

client.println();

client.println("<!DOCTYPE html>");

client.println("<html>");

client.println("<head><title>ESP32-CAM Live Stream</title></head>");

client.println("<body style='text-align:center;'>");

client.println("<h1>ESP32-CAM Live Feed</h1>");

client.println("<img src='/stream' style='max-width:100%; height:auto;'>");

client.println("</body>");

client.println("</html>");

}

// Give the client time to receive the data

delay(1);

Serial.println("Client Disconnected.");

}