Simple motor front back

// ESP32 Tank Robot with L9110 Motor Driver

// Sequence: Forward 5s → Back 5s → Right 90° → Left 90° → Repeat

// L9110 pins: B-IA, B-IB = Left motor | A-IA, A-IB = Right motor

// Left Motor (L9110 Channel B)

#define LEFT_IA   12   // Motor direction pin 1

#define LEFT_IB   14   // Motor direction pin 2

// Right Motor (L9110 Channel A)

#define RIGHT_IA  26   // Motor direction pin 1

#define RIGHT_IB  27   // Motor direction pin 2

// Speed pins? L9110 doesn't need PWM pins - just use digital pins with delay

// But for speed control, we'll use any PWM-capable pins (here using same pins)

int motorSpeed = 200;      // PWM value 0-255 (speed for forward/back)

int turnSpeed = 180;       // Speed during turns (lower = smoother)

int turnTime = 800;        // Milliseconds for 90° turn (adjust this!)

void setup() {

Serial.begin(115200);

pinMode(LEFT_IA, OUTPUT);

pinMode(LEFT_IB, OUTPUT);

pinMode(RIGHT_IA, OUTPUT);

pinMode(RIGHT_IB, OUTPUT);

// Configure PWM on all pins (ESP32 requires this for analogWrite)

ledcSetup(0, 5000, 8);   // channel 0, 5kHz, 8-bit

ledcAttachPin(LEFT_IA, 0);

ledcSetup(1, 5000, 8);

ledcAttachPin(LEFT_IB, 1);

ledcSetup(2, 5000, 8);

ledcAttachPin(RIGHT_IA, 2);

ledcSetup(3, 5000, 8);

ledcAttachPin(RIGHT_IB, 3);

Serial.println("Tank Robot Ready!");

Serial.println("Sequence: FORWARD 5s → BACK 5s → RIGHT 90° → LEFT 90°");

delay(2000);

}

void loop() {

forward(5);      // Move forward for 5 seconds

stop(1);         // Pause 1 second

backward(5);     // Move backward for 5 seconds

stop(1);

turnRight();     // Turn right 90°

stop(1);

turnLeft();      // Turn left 90°

stop(1);

}

// ========== Motion Functions (L9110) ==========

void forward(int seconds) {

Serial.println("→ FORWARD");

// Left motor forward

analogWrite(LEFT_IA, motorSpeed);

analogWrite(LEFT_IB, 0);

// Right motor forward

analogWrite(RIGHT_IA, motorSpeed);

analogWrite(RIGHT_IB, 0);

delay(seconds * 1000);

}

void backward(int seconds) {

Serial.println("← BACKWARD");

// Left motor backward

analogWrite(LEFT_IA, 0);

analogWrite(LEFT_IB, motorSpeed);

// Right motor backward

analogWrite(RIGHT_IA, 0);

analogWrite(RIGHT_IB, motorSpeed);

delay(seconds * 1000);

}

void turnRight() {

Serial.println("↻ TURN RIGHT 90°");

// Pivot right: left motor forward, right motor backward

analogWrite(LEFT_IA, turnSpeed);

analogWrite(LEFT_IB, 0);

analogWrite(RIGHT_IA, 0);

analogWrite(RIGHT_IB, turnSpeed);

delay(turnTime);

}

void turnLeft() {

Serial.println("↺ TURN LEFT 90°");

// Pivot left: left motor backward, right motor forward

analogWrite(LEFT_IA, 0);

analogWrite(LEFT_IB, turnSpeed);

analogWrite(RIGHT_IA, turnSpeed);

analogWrite(RIGHT_IB, 0);

delay(turnTime);

}

void stop(int seconds) {

Serial.println("■ STOP");

analogWrite(LEFT_IA, 0);

analogWrite(LEFT_IB, 0);

analogWrite(RIGHT_IA, 0);

analogWrite(RIGHT_IB, 0);

delay(seconds * 1000);

}